Fauber: Radiographic Imaging and Exposure, 4th Edition

Chapter 01: Radiation and Its Discover

Test Bank

MULTIPLE CHOICE

1.	When were x-rays discovered?

| a. | October 8, 1985 |
| --- | --- |
| b. | November 8, 1895 |
| c. | January 23, 1896 |
| d. | August 15, 1902 |

ANS:	B

X-rays were discovered by Wilhelm Conrad Roentgen on November 8, 1895.

REF:	2	OBJ:	3

2.	What type of tube was Roentgen working with in his lab when x-rays were discovered?

| a. | Crookes tube |
| --- | --- |
| b. | Fluorescent tube |
| c. | High-vacuum tube |
| d. | Wurzburg tube |

ANS:	A

Roentgen was working with a low-vacuum tube known as a Crookes tube.

REF:	2	OBJ:	3

3.	Which of the following terms could be defined as the instantaneous production of light only during an interaction between a type of energy and some element or compound?

| a. | Phosphorescence |
| --- | --- |
| b. | Afterglow |
| c. | Glowing |
| d. | Fluorescence |

ANS:	D

Fluorescence is the instantaneous emission of light from a material due to the interaction with some type of energy.

REF:	3	OBJ:	3

4.	Barium platinocyanide was the:

| a. | Type of dark paper Roentgen used to darken his laboratory |
| --- | --- |
| b. | Material Roentgen used to produce the first radiograph of his wife's hand |
| c. | Metal used to produce the low-vacuum tube |
| d. | Fluorescent material that glowed when the tube was energized |

ANS:	D

A piece of paper coated with barium platinocyanide glowed each time Roentgen energized his tube.

REF:	2-3	OBJ:	3

5.	The first radiograph produced by Roentgen, of his wife's hand, required an exposure time of:

| a. | 15 seconds |
| --- | --- |
| b. | 150 seconds |
| c. | 15 minutes |
| d. | 150 minutes |

ANS:	C

It took a 15-minute exposure time to produce the first radiograph.

REF:	3	OBJ:	3

6.	The letter x in x-ray is the symbol for:

| a. | Electricity |
| --- | --- |
| b. | The unknown |
| c. | Penetrating |
| d. | Discovery |

ANS:	B

The letter x represents the mathematical symbol of the unknown.

REF:	3	OBJ:	3

7.	The first Nobel Prize for physics was received in 1901 by:

| a. | Marie Curie |
| --- | --- |
| b. | Crookes |
| c. | Wilhelm Roentgen |
| d. | Einstein |

ANS:	C

Wilhelm Roentgen received the first Nobel Prize for physics in 1901.

REF:	5	OBJ:	3

8.	Erythema, an early sign of biological damage due to x-ray exposure, is:

| a. | Reddening of the skin |
| --- | --- |
| b. | A malignant tumor |
| c. | A chromosomal change |
| d. | One of the most serious effects of x-ray exposure |

ANS:	A

Erythema is reddening and burning of the skin, an early and less serious effect of exposure to large doses of x-radiation.

REF:	5	OBJ:	3

9.	X-rays have which of the following properties?

| a. | Electrical properties |
| --- | --- |
| b. | Magnetic properties |
| c. | Chemical properties |
| d. | A and B |

ANS:	D

X-rays, a type of electromagnetic radiation, have both electrical and magnetic properties.

REF:	5	OBJ:	4

10.	The distance between two successive crests of a sine wave is known as:

| a. | An angstrom |
| --- | --- |
| b. | Frequency |
| c. | The Greek letter nu |
| d. | Wavelength |

ANS:	D

The distance between two successive crests or troughs of a sine wave is the measure of its wavelength.

REF:	6	OBJ:	4

11.	X-rays used in radiography have wavelengths that are measured in:

| a. | Angstroms |
| --- | --- |
| b. | Millimeters |
| c. | Centimeters |
| d. | Inches |

ANS:	A

X-rays in the range used in radiography have wavelengths that are so short that they are measured in angstroms.

REF:	6	OBJ:	4

12.	The frequency of a wave is the number of waves passing a given point per given unit of time. Frequency is measured in:

| a. | Angstroms |
| --- | --- |
| b. | Hertz |
| c. | Inches |
| d. | eV |

ANS:	B

The unit of frequency is Hertz. The frequency of x-rays in the radiography range varies from about 3  1019 to 3  1018 Hz.

REF:	6	OBJ:	4

13.	Which of the following is a correct description of the relationship between the wavelength and frequency of the x-ray photon?

| a. | Wavelength and frequency are directly proportional. |
| --- | --- |
| b. | Wavelength and frequency are inversely related by the square root of lambda. |
| c. | Frequency and wavelength are inversely related. |
| d. | Wavelength and frequency have no relationship to each other. |

ANS:	C

Wavelength and frequency are inversely related; as one increases, the other decreases.

REF:	7	OBJ:	4

14.	A _____ is a small, discrete bundle of energy.

| a. | Phaser |
| --- | --- |
| b. | Quark |
| c. | Photon |
| d. | Mesion |

ANS:	C

A photon, or quantum, is a small, discrete bundle of energy.

REF:	7	OBJ:	4

15.	The speed of light is:

| a. | 3  108 meters per second |
| --- | --- |
| b. | 3  108 miles per second |
| c. | 186,000 miles per second |
| d. | A and C |

ANS:	D

The speed of light can be described as either 3  108 meters per second or 186,000 miles per second.

REF:	7	OBJ:	4

16.	Which of the following were reactions to the discovery of x-rays?

| a. | Development of x-ray proof underwear |
| --- | --- |
| b. | Laws passed to ban x-ray producing devices in opera glasses |
| c. | Recommendation to execute Roentgen |
| d. | All of the above |

ANS:	D

Soon after Roentgen's discovery, x-ray proof underwear, a recommendation to have Roentgen executed, and a law to ban x-ray producing devices in glasses came about.

REF:	4-5	OBJ:	3

17.	When first developed, the branch of medicine using x-rays was called:

| a. | Radiology |
| --- | --- |
| b. | Radiography |
| c. | Roentgenology |
| d. | Imaging sciences |

ANS:	C

What we now call radiology was first called Roentgenology.

REF:	5	OBJ:	3

18.	The electrical energy applied to an x-ray tube will be transformed to:

| a. | Heat |
| --- | --- |
| b. | Light |
| c. | X-rays |
| d. | A and C |

ANS:	D

The electrical energy applied to the tube will be transformed into heat (primarily) and x-rays.

REF:	5	OBJ:	4

19.	The Greek symbol lambda () represents the x-ray's:

| a. | Wavelength |
| --- | --- |
| b. | Speed |
| c. | Frequency |
| d. | Quantity |

ANS:	A

Lambda () is the Greek symbol that represents wavelength.

REF:	6	OBJ:	5

20.	An angstrom (Å) is equal to:

| a. | 10-1 meter |
| --- | --- |
| b. | 10-10 meter |
| c. | 10-1 foot |
| d. | 10-10 foot |

ANS:	B

One angstrom is equal to 10-10 meter.

REF:	6	OBJ:	5

21.	X-rays used in radiography have wavelengths ranging from 0.1 Å to:

| a. | 0.01 Å |
| --- | --- |
| b. | 1 Å |
| c. | 10 Å |
| d. | 100 Å |

ANS:	B

X-rays used in radiography have wavelengths ranging from 0.1 Å to 1 Å.

REF:	6	OBJ:	5

22.	In the formula c = v, c represents:

| a. | Frequency |
| --- | --- |
| b. | The speed of light |
| c. | Wavelength |
| d. | Kinetic energy |

ANS:	B

In this formula, c represents the speed of light.

REF:	7	OBJ:	5

23.	In the formula c = v, v represents:

| a. | Frequency |
| --- | --- |
| b. | The speed of light |
| c. | Wavelength |
| d. | Kinetic energy |

ANS:	A

In this formula, v represents frequency.

REF:	7	OBJ:	5

24.	The energy of an individual x-ray photon is measured in:

| a. | Frequency |
| --- | --- |
| b. | Wavelength |
| c. | Kilovolts peak (kVp) |
| d. | Electron volts (eV) |

ANS:	D

X-ray photon energy is measured in electron volts (eV).

REF:	8	OBJ:	5

25.	An x-ray beam that has photons with many different energies is:

| a. | Homogenous |
| --- | --- |
| b. | Monoenergetic |
| c. | Heterogeneous |
| d. | Never found |

ANS:	C

A heterogenous x-ray beam consists of photons with many different energies.

REF:	8	OBJ:	6

26.	X-rays can:

| a. | Penetrate the human body |
| --- | --- |
| b. | Be absorbed in the human body |
| c. | Change direction in the human body |
| d. | All of the above |

ANS:	D

X-rays can penetrate, be absorbed in, or change direction (due to scattering) in the human body.

REF:	9	OBJ:	6

TRUE/FALSE

1.	X-rays are invisible.

A. True

B. False

ANS:	T

A characteristic of x-rays is that they are invisible.

REF:	8	OBJ:	6

2.	X-rays carry a negative charge that causes ionization.

A. True

B. False

ANS:	F

X-rays are electrically neutral.

REF:	8	OBJ:	6

3.	X-ray photons travel at the speed of light in a vacuum.

A. True

B. False

ANS:	T

In a vacuum x-rays will travel at the speed of light.

REF:	8	OBJ:	6

4.	X-ray photons are capable of traveling around corners.

A. True

B. False

ANS:	F

X-rays travel in straight lines, so they are unable to travel around corners.

REF:	9	OBJ:	6

5.	Chemical changes may occur as a result of exposure to ionizing radiation.

A. True

B. False

ANS:	T

Chemical changes, such as in radiographic or photographic film, occur as a result of exposure to ionizing radiation.

REF:	8-9	OBJ:	6

6.	X-rays will change direction in the presence of a strong magnetic field.

A. True

B. False

ANS:	F

X-rays do not respond to a magnetic field.

REF:	8	OBJ:	6

7.	X-rays produce a slight tingling sensation when they enter the body.

A. True

B. False

ANS:	F

X-rays cannot be felt.

REF:	8	OBJ:	6

8.	X-rays cannot be focused with a lens.

A. True

B. False

ANS:	T

Unlike visible light, it is not possible to focus x-rays with a lens.

REF:	8	OBJ:	6

9.	X-rays are able to interact with certain materials and produce light energy.

A. True

B. False

ANS:	T

Certain materials will fluoresce, or produce light energy, when stimulated by x-rays.

REF:	9	OBJ:	6

10.	It is impossible for x-rays to interact with matter and produce secondary radiation.

A. True

B. False

ANS:	F

Secondary radiation is often produced as a result of x-rays interacting with matter.

REF:	9	OBJ:	6

11.	X-rays can produce ionization of atoms making up cells, causing damage.

A. True

B. False

ANS:	T

A major reason that unnecessary exposure must be avoided is that x-rays can ionize atoms and cause damage.

REF:	9	OBJ:	6

12.	Since Roentgen's discovery in the late nineteenth century, we have learned an enormous amount about the properties of x-rays.

A. True

B. False

ANS:	F

Roentgen's original work on the characteristics of x-rays was so thorough that very little has been learned about their properties since.

REF:	9	OBJ:	3